Wizard SV Gel and PCR Clean-Up System

If you have PCR product that either has multiple bands, or a single band with smearing, you will need to gel purify the target band (most of the time the largest, but depends on your target size).  To do this:

1) Run remaining 20 µL of PCR product on gel with large wells (often use the large gel with the 25 well comb).  Run for at least an hour (120 V small gel and 300 V large gel) to get good band separation.

2) Stain gel in Syber gold solution for 30 minutes.

3) Cut out target bands, getting as little extra agarose around the cut as possible.

4) Put cut bands in eppendorf tube, and weigh (important for next step).

5) Add in appropriate amount of membrane binding solution and incubate at 55°C for 10 minutes or until completely dissolved.  Most tubes weigh 1 gram, so if your tube + band weighs 1.26 grams, then that is a 260 mg difference and you will add in 260 µL of membrane binding solution.

6) For each gel slice, place one SV Minicolumn into a Collection tube.

7) Transfer the dissolved gel mixture to the SV column and incubate at room temperature for 1 minute.

8) Centrifuge the column at max speed (13.2 rpm) for 1 minute.  Discard liquid in the collection tube.

9) Wash the column by adding in 700 µL of Membrane Wash Solution, spin for 1 minute. Discard waste.

10) Add in 500 µL of Wash solution, spin for 5 minutes.  Discard waste.

11) Spin the column again for 1 minute without adding any more liquid (this ensures that all the ethanol is gone).

12) Transfer column to a clean, well labeled eppendorf tube.  Add in 30 µL of H20, incubate at room temperature 1 minute, then centrifuge for 1 minute.

13) Discard column while keeping the liquid (this is what you were aiming to get).  Place in freezer or fridge until adding to sequencing plate.