Gel making and running

- Normally run a 1.5% agarose gel.  For the small rigs you need about 100 mL and for the large gel you need about 300 mL.  For each 100 mLs of TBE buffer you need to add in 1.5 grams of agarose.

- Combine buffer and agarose powder into appropriate sized flask.  Microwave between 1 and 2 minutes.  You want the liquid to be clear and not be able to see any bits of agarose powder.

- Set up gel holder in pouring rig, and then pour in gel.  Place appropriate combs.  Gel should take between 15-20 minutes to solidify and be ready to run.

- When gel is ready to use, transfer gel holder and gel into gel rig.  You want to have about a quarter inch or so of liquid above the gel (entire gel should be submerged).

- Load in 3 µL of ladder and 5 µL of PCR product in wells.

- For the small gels, run at 120 V for about 35 minutes (You want the yellow color to be at least halfway down the given area you are running)

- Once gel is done, transport it to gel staining tub.  Let it sit there for 20 minutes.

- Transfer gel to destaining tray of just water.  Leave in there for 10 minutes.

- When done, put on UV plate and close the lid.

- Sign on to the computer using GatorLink, and open the PC Image program

- Make sure the appropriate camera is selected, center the gel and hit expose.

- Save the file to the server.  To get onto server, open the Computer tab under start preference, and click map server.  Type in \\farnsworth.flmnh.ufl.edu\soltis\Molecular\Polemoniaceae\Gel_images       Saving the gels here will allow everyone to access them from any computer.  Save the file with the gene, date and your initials (ex: trnLtrnF_062212_JBL.tiff)

- Print the image to place in lab notebook.

- When done, log off computer, throw the gel away into white bins, and wipe down surface of imager.

Total time should take somewhere between 1 hour and 20 minutes and 1 hour and 45 minutes