Library Prep with User Supplied Primers and Adaptors

Starting Material: 1-5 g of Fragmented DNA

Before starting End Repair, take Beads out of Fridge

End Repair of Fragmented DNA

1. Mix the following components in a PCR tube

NEBNext End Repair Reaction Buffer (10X)	5 L

NEBNext End Repair Enzyme Mix			2.5 L

Fragmented DNA (between 3-5g)			20 L

If concentration more than 10, use half

Sterile water						22.5 L

Total							50 L

2. Incubate in thermal cycler for 30 minutes at 20C.  Lid at 45C

Clean Up Using AMPure/Axygen Beads

1.  Transfer Ligation reaction product to clean eppendorf tube.

2. Vortex beads to resuspend.

3. Add 90 L (1.8 volume) of resuspended beads to the ligation reaction.  Mix thoroughly by pipetting up and down 10 times.

4. Incubate at room temperature for 5 minutes.

5.  Put tubes in magnetic stand to separate beads from supernatant.  After the solution is clear (about 5 minutes), carefully remove and discard the supernatant from all tubes.  Be careful not to disturb the beads that contain the DNA targets.

6.  Add 180 L of 80% freshly made ethanol.  Pipette up and down slowly 5 times without disturbing beads, then remove supernatant.  Repeat for each tube (Wash each tube twice, before moving on to the next).

7.  Air dry beads for 10 minutes while the tubes are still in the stand.  Beads should no longer have a shine to them, and no ethanol should be visible.

8. Elute target DNA by adding 22 L sterile water to the beads.  Mix well with vortex until all beads are suspended.  Place tubes back on magnetic stand.

9.  When solution is clear, carefully remove 21 L and place in PCR tube for next step.

dA-Tailing of End Repaired DNA

1. Mix the following components in the PCR tube prepared during the previous step:

End Repaired Blunt DNA				21 L

NEBNext dA-Tailing Reaction Buffer (10x)		2.5 L

Klenow Fragment (3’  5 exo)			1.5 L

Total							25 L

2. Incubate in thermocycler fro 30 minutes at 37C.  Lid at 60C.

Clean using AMPure/Axygen beads

1. Transfer product from above into clean eppendorf tube.

2. Vortex beads to make sure suspended.

3. Add 90 L (1.8 volume) of beads to tube.  Mix thoroughly by pipetting up and down at least 10 times.

4. Incubate at room temperature for 5 minutes.

5.  Put tubes in magnetic stand to separate beads from supernatant.  After the solution is clear (about 5 minutes), carefully remove and discard the supernatant from all tubes.  Be careful not to disturb the beads that contain the DNA targets.

6.  Add 180 L of 80% freshly made ethanol.  Pipette up and down slowly 5 times without disturbing beads, then remove supernatant.  Repeat for each tube (Wash each tube twice, before moving on to the next).

7.  Air dry beads for 10 minutes while the tubes are still in the stand.  Beads should no longer have a shine to them, and no ethanol should be visible.

8. Elute target DNA by adding 16 L of sterile water to beads.  Mix well with vortex until all beads are suspended.  Place tubes back on magnetic stand.

9.  When solution is clear, carefully remove 15 L and place in PCR tube for next step.

Adaptor Ligation of dA-Tailed DNA

1. Mix the following components into the PCR tube prepared in the previous step:

dA-Tailed DNA					25 L

Quick Ligation Reaction Buffer (5X)		5 L

15 M DNA adaptors/barcodes			2.5 L

Quick T4 DNA Ligase					2.5 L

Total Volume						25 L

2. Incubate in thermocycler for 15 mintes at 20C.  Lid at 45C.

3. While reaction is occurring, fill in Nextflex_Barcode_Usage.xlsx found on server in NextGenSeq with which barcodes were used.

4. Fill in spreadsheet LibraryPrep.xlsx found inPolemoniaceaeMybaits folder.

Clean using AMPure/Axygen beads

1. Transfer product from above into clean eppendorf tube.

2. Vortex beads to make sure suspended.

3. Add 45 L (1.8 volume) of beads to tube.  Mix thoroughly by pipetting up and down at least 10 times.

4. Incubate at room temperature for 5 minutes.

5.  Put tubes in magnetic stand to separate beads from supernatant.  After the solution is clear (about 5 minutes), carefully remove and discard the supernatant from all tubes.  Be careful not to disturb the beads that contain the DNA targets.

6.  Add 180 L of 80% freshly made ethanol.  Pipette up and down slowly 5 times without disturbing beads, then remove supernatant.  Repeat for each tube (Wash each tube twice, before moving on to the next).

7.  Air dry beads for 10 minutes while the tubes are still in the stand.  Beads should no longer have a shine to them, and no ethanol should be visible.

8. Elute target DNA by adding 26 L of sterile water to beads.  Mix well with vortex until all beads are suspended.  Place tubes back on magnetic stand.

9.  When solution is clear, carefully remove 25 L and place in clean eppendorf for next step.  Also, add additional 25 L of sterile water to bring total volume up to 50 L.

Size Selector Adaptor Ligated DNA Using AMPure/Axygen beads

1.  Add 40 L (0.8 volume) of beads to tube.  Mix thoroughly by pipetting up and down at least 10 times.

4. Incubate at room temperature for 5 minutes.

5.  Put tubes in magnetic stand to separate beads from supernatant.  After the solution is clear (about 5 minutes), carefully remove and discard the supernatant from all tubes.  Be careful not to disturb the beads that contain the DNA targets.

6.  Add 180 L of 80% freshly made ethanol.  Pipette up and down slowly 5 times without disturbing beads, then remove supernatant.  Repeat for each tube (Wash each tube twice, before moving on to the next).

7.  Air dry beads for 10 minutes while the tubes are still in the stand.  Beads should no longer have a shine to them, and no ethanol should be visible.

8. Elute target DNA by adding 25 L of sterile water to beads.  Mix well with vortex until all beads are suspended.  Place tubes back on magnetic stand.

9.  When solution is clear, carefully remove 25 L and place in clean tube for storage.

PCR Enrichment of Adaptor Ligated DNA

1. Mix the following components in a sterile PCR tube:

Size selected DNA					10 L

Primer 1 (25 M)					1.25 L

Primer 2 (25 M)					1.25 L

NEBNext High Fidelity 2X PCR Master Mix		12.5 L

Total Volume						25 L

2. PCR cycling conditions

Initial Denaturation 98C for 30 seconds

4-8 cycles of Denaturation 98C for 10 seconds

Annealing 65C for 30 seconds

Extension 72C for 30 seconds

Final Extension 72C for 5 minutes

Hold 4C hold

Additional Cleanup using AMPure/Axygen beads

1.  Add 40 L (0.8 volume) of beads to tube.  Mix thoroughly by pipetting up and down at least 10 times.

4. Incubate at room temperature for 5 minutes.

5.  Put tubes in magnetic stand to separate beads from supernatant.  After the solution is clear (about 5 minutes), carefully remove and discard the supernatant from all tubes.  Be careful not to disturb the beads that contain the DNA targets.

6.  Add 180 L of 80% freshly made ethanol.  Pipette up and down slowly 5 times without disturbing beads, then remove supernatant.  Repeat for each tube (Wash each tube twice, before moving on to the next).

7.  Air dry beads for 10 minutes while the tubes are still in the stand.  Beads should no longer have a shine to them, and no ethanol should be visible.

8. Elute target DNA by adding 26 L of sterile water to beads.  Mix well with vortex until all beads are suspended.  Place tubes back on magnetic stand.

9.  When solution is clear, carefully remove 25 L and place in clean tube for storage.